Maritime trade corridor and inland linkage map - Ghana

Maritime trade in Ghana flows through a coastal gateway network that only generates its full economic value when cargo can reach inland markets via road and rail. Planners, logistics operators, and infrastructure investors currently lack a spatially explicit map showing how far that connectivity actually extends from each gateway. You are required to perform a trade corridor connectivity analysis that delineates the inland reach of Ghana's maritime gateways at three distance thresholds, producing a decision-ready spatial dataset showing which road and rail assets serve those corridors.

You will be working with four layers: sea_ports.gpkg (46 point features covering ferry terminals and maritime facilities), ports.gpkg (5 point features covering major commercial ports including Takoradi, Tema, Buipe, and Akosombo), roads.gpkg (374,753 line features of the national road network), and rail_pip.gpkg (15 line features of rail and pipeline corridors). All layers share the same projected CRS. No reprojection is required.

The first derived output is maritime_gateways.gpkg, a clean 51-feature point layer consolidating both input gateway datasets. It carries three fields: gateway_name (drawn from the name or amenity field for sea_ports features, and from MAIN_PORT_ or Original for ports features), gateway_type (the amenity value in title-case for sea_ports, and the string Major Port for all ports features), and source_layer (sea_ports or ports respectively). The second output, gateway_corridor_buffers.gpkg, is generated by applying a Multi-Ring Constant Buffer to maritime_gateways.gpkg with a step distance of 50,000 metres and a ring count of 3. This produces 153 buffer polygons each carrying a ringId field (1, 2, or 3) and a distance field (50,000, 100,000, 150,000), representing Zone 1 (0-50 km), Zone 2 (50-100 km), and Zone 3 (100-150 km) from any gateway. The 153 buffers are then dissolved into a single polygon to serve as the spatial extraction mask. roads_within_corridors.gpkg is produced by extracting all features from roads.gpkg that intersect the dissolved mask using Extract by Location with the intersects predicate, retaining full geometry. rail_within_corridors.gpkg is produced by clipping rail_pip.gpkg to the same dissolved mask.

In the project, gateway_corridor_buffers.gpkg is symbolised using a categorised renderer on ringId: Zone 1 carries dark navy fill (#0B3D91, alpha 50) with matching stroke; Zone 2 carries mid-blue fill (#1565C0, alpha 35); Zone 3 carries light blue fill (#42A5F5, alpha 25). roads_within_corridors.gpkg is rendered as a single amber line (#E67E00, 0.25 mm). rail_within_corridors.gpkg is rendered as a dark red line (#8B0000, 1.0 mm). maritime_gateways.gpkg is symbolised with a categorised renderer on gateway_type: Major Port features render as red diamond markers at size 8 with white outline; Ferry Terminal features render as blue circles at size 5; all remaining types render as grey circles at size 4.

The map output is an A4 landscape layout named Maritime_Trade_Corridor_and_Inland_Linkage_Map, exported as Maritime_Trade_Corridor_and_Inland_Linkage_Map.pdf at 300 DPI. Map extent covers the combined extent of maritime_gateways.gpkg and gateway_corridor_buffers.gpkg. The layout carries the title Maritime Trade Corridors and Inland Linkage Map - Ghana, a legend covering all four zone labels and all gateway type symbols, a north arrow, a scale bar in kilometres, a CRS note, and a data source note naming all four input layers.

Deliver the project folder Maritime trade corridor and inland linkage map containing the four input layers, all four derived outputs, the PDF, and Maritime_Trade_Corridor_and_Inland_Linkage_Map.qgz saved with relative paths so the project reopens without broken layer links, CRS mismatches, or missing files.